Experiment Set 1: Benefits of Long-Term Planning (PL)

Objective

Show that intelligent topology selection reduces OPEX while maintaining SLA.

Compare

| Scheme | PL | PM | PS |
| --- | --- | --- | --- |
| Full Algorithm | ✓ | ✓ | ✓ |
| Fixed Edge | ✗ | ✓ | ✓ |
| Fixed Regional | ✗ | ✓ | ✓ |
| Fixed Central | ✗ | ✓ | ✓ |

Scenario

Mix of slices:

- eMBB

- URLLC

- Video Analytics

- Cloud Gaming

with different latency constraints.

Allow PL to choose:

- Edge

- Regional

- Central

for

- CU-UP

- UPF

- APP

Metrics

OPEX

Ctotal

SLA satisfaction

PSLA met

Average E2E delay

DE2E

Expected Figure

Figure A

OPEX vs SLA target

99.99%

99.9%

99%

95%

Result:

- Fixed Edge → high OPEX

- Fixed Central → SLA failures

- PL → Pareto-optimal

Key Message

PL intelligently places functions near or far from users according to SLA requirements.

Experiment Set 2: Benefits of Medium-Term Scaling (PM)

Current paper already shows some effects indirectly.

Journal version should isolate PM.

Objective

Show benefit of dynamic compute scaling.

Compare

| Scheme | PL | PM | PS |
| --- | --- | --- | --- |
| Full | ✓ | ✓ | ✓ |
| No PM | ✓ | ✗ | ✓ |

No PM:

- CPU

- RAM

- GPU

fixed after admission.

Dynamic Traffic Scenario

Use realistic daily pattern:

Office hours

08:00-12:00

traffic = 100%

Lunch

12:00-14:00

traffic = 40%

Afternoon

14:00-18:00

traffic = 120%

Night

traffic = 20%

Run 24-hour simulation.

Metrics

Resource utilization

UCPU

Resource wastage

W=Reserved-Used

OPEX

Queue backlog

Packet latency

Expected Figure

CPU allocation

No PM:

=====================

fixed allocation

PM:

/\/\/\/\/\/\

follows demand

Expected Outcome

PM yields:

- 25-40% OPEX reduction

- reduced idle resources

- lower queue growth during bursts

Additional Experiment

Traffic forecast error:

0%, 10%, 20%, 30%, 50%

Demonstrates robustness.

Experiment Set 3: Benefits of Short-Term Scheduling (PS)

This is probably where the strongest gains will appear.

Objective

Show adaptation to fast channel variations.

Compare

| Scheme | PL | PM | PS |
| --- | --- | --- | --- |
| Full | ✓ | ✓ | ✓ |
| No PS | ✓ | ✓ | ✗ |

No PS:

Fixed PRB allocation.

Scenario

Emulate realistic radio conditions.

User mobility

- Stationary

- Walking

- Vehicle

- UAV

Generate changing:

- CQI

- MCS

- Spectral efficiency

using your Eq. (12). [Assuring_S...tem_Design | PDF]

Metrics

Throughput

T

SLA violation

V

Cell-edge throughput

PRB utilization

Expected Figure

Time series:

Spectral Efficiency

/\/\_/\/\/\

Fixed PRB

----------

Throughput drops

Dynamic PS

^^^^^^^^^^

maintains SLA

Additional Scenario

Deep fading event

At t=100s:

η→0.5η

for 30 seconds.

Measure:

Recovery time

Trecover

Show PS rapidly reallocates PRBs.

Experiment Set 4: Synergy Between PL-PM-PS

This is the most publishable figure.

Instead of only comparing against static baseline, compare:

| Scheme | PL | PM | PS |
| --- | --- | --- | --- |
| S0 | ✗ | ✗ | ✗ |
| S1 | ✓ | ✗ | ✗ |
| S2 | ✓ | ✓ | ✗ |
| S3 | ✓ | ✓ | ✓ |

Metrics

SLA violation

OPEX

Throughput

Resource efficiency

Waterfall Plot

Example:

SLA Violation Reduction

Static            100%

+ PL               65%

+ PM               35%

+ PS               10%

OPEX

Static            100%

+ PL               80%

+ PM               55%

+ PS               50%

This directly quantifies the contribution of each layer.

Strongest Journal Figure

I would recommend the following summary figure:

| Control Layer | Primary Benefit | Metric |
| --- | --- | --- |
| PL | Optimal placement | OPEX, Latency |
| PM | Elastic compute scaling | CPU utilization, Queue backlog |
| PS | Channel adaptation | SLA violation, Throughput |
| PL+PM+PS | End-to-end optimization | SLA + OPEX Pareto frontier |

This figure will clearly convince reviewers that PL handles where resources are deployed, PM handles how much compute is allocated, and PS handles how radio resources react to channel dynamics, and that all three layers are necessary to achieve the gains shown in the paper.